ENVIRONMENTAL & SAFETY MONITORING NODE: HARDWARE SETUP MANUAL

1. Project Bill of Materials (BOM)

The following components are utilized in the construction of the stationary environmental checkpoint monitor.

| Item | Component | Specification | Qty | Target Function |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 DevKit V1 (30-pin) | 1 | Processes edge data, executes ISR, handles Wi-Fi payload transmission. |
| 2 | Environmental Sensor | DHT22 | 1 | Captures ambient temperature (°C) and relative humidity (%). |
| 3 | Emergency Switch | 6x6mm Tactile Push Button | 1 | Triggers the high-priority OS emergency daemon alert. |
| 4 | Prototyping Board | MB102 Solderless Breadboard | 1 | Provides the physical platform to mount and wire all components. |
| 5 | Interconnects | Solid-core Jumper Wires (M-M) | 6 | Links power, ground, and logic lines between devices. |

2. Definitive Pin Mapping Table

To prevent shorts and ensure the hardware interacts flawlessly with the compiled firmware binaries, components must be wired exactly according to the mapping table below:

| From Component | Component Pin | To ESP32 Pin | Logic Type |
| --- | --- | --- | --- |
| DHT22 Sensor | VCC (Pin 1) | 3.3V | Power |
|  | DATA (Pin 2) | GPIO 4 | Digital Input |
|  | GND (Pin 3) | GND | Ground |
| Tactile Button | Terminal 1A | GPIO 15 | Digital Input |
|  | Terminal 1B | GND | Ground |